# SECTION 01045 CUTTING AND PATCHING

## PART 1- GENERAL

### REQUIREMENTS INCLUDED

    - Contractor shall be responsible for all cutting, fitting and patching, including related excavation and backfill, required to complete the Work or to:

      - Make its several parts fit together properly.

      - Uncover portions of the Work to provide for installation of ill-timed work.

      - Remove and replace defective work.

      - Remove and replace work not conforming to requirement of Contract Documents.

      - Remove samples of installed work as specified for testing.

      - Provide routine penetrations of non-structural surfaces for installation of piping and electrical conduit.

  - RELATED REQUIREMENTS

    - Section 01010:  Summary of Work.

    - Section 01630:  Substitutions and Product Options.

    - Section 02220:  Trenching, Backfilling and Compacting.

    - Section 02661:  Water Mains.

  - SUBMITTALS

    - Submit a written request to Engineer well in advance of executing and cutting or alteration which affects:

      - Work of the Owner or any separate contractor.

      - Structural value or integrity of any element of the project.

      - Integrity or effectiveness of weather-exposed or moisture-resistant element  or systems.

      - Efficiency, operational life, maintenance or safety of operational elements.

      - Visual qualities of sight-exposed elements.

    - Request shall include:

### Identification of the Project.

      - Description of affected work.

      - The necessity for cutting, alteration or excavation.

      - Effect on work of Owner or any separate contractor, or on structural or weatherproof integrity of Project.

      - Description of proposed work:

        - Scope of cutting, patching, alteration, or excavation.

        - Trades who will execute the work.

        - Products proposed to be used.

        - Extent of refinishing to be done.

      - Alternative to cutting and patching.

      - Cost proposal, when applicable.

      - Written permission of any separate contractor whose work will be affected.

    - Should conditions of Work or the schedule indicate a change of products from original installation, contractor shall submit request for substitution as specified in Section 01630 - Substitutions and Product Options.

    - Submit written notice to Engineer designating the date and the time the Work will be uncovered.

PART 2 - PRODUCTS

  - MATERIALS

    - Comply with specifications and standards for each specific product involved.

PART 3 - EXECUTION

  - INSPECTION

    - Inspect existing conditions of Project, including elements subject to damage or to movement during cutting and patching.

    - After uncovering work, inspect conditions affecting installation of Products, or performance of work.

    - Report unsatisfactory or questionable conditions to Engineer in writing; do not proceed with work until Engineer has provided further instructions.

  - PREPARATION

    - Provide adequate temporary support as necessary to assure structural value or integrity of affected portion of work.

    - Provide devices and methods to protect other portions of Project from damage.

    - Provide protection from elements for that portion of the Project which may be exposed by cutting and patching work, and maintain excavations free from water.

  - PERFORMANCE

    - Execute cutting and demolition by methods which will prevent damage to other work, and will provide proper surfaces to receive installation of repairs.

    - Execute excavating and backfilling by methods which will prevent settlement or damage to other work.

    - Employ original Installer or Fabricator to perform cutting and patching for:

      - Weather-exposed or moisture-resistant elements.

      - Sight-exposed finished surfaces.

    - Execute fittings and adjustment of products to provide a finished installation to comply with specified products, functions, tolerances and finishes.

    - Restore work which has been cut or removed; install new products to provide completed work in accord with requirements of Contract documents.

    - Fit work airtight to pipes, sleeves, ducts, conduit or other penetrations through surfaces.

    - Refinish entire surfaces as necessary to provide an even finish to match adjacent finishes;

      - For continuous surfaces, refinish to nearest intersection.

      - For an assembly, refinish entire unit.

END OF SECTION 01045